DIGIEQUINE V3

RTOS Firmware Engineering Handbook

From requirements and architecture to drivers, tasks, interrupts, testing, and interview reasoning

Purpose
This handbook explains how the V3 firmware was designed and built, why each RTOS strategy was selected, how the code behaves, and how to discuss the engineering trade-offs honestly in a technical interview.

Target: ESP32-S3 | Framework: ESP-IDF | Kernel: FreeRTOS

Document date: June 4, 2026

# How to Use This Handbook

Read Chapters 1-4 first if RTOS is new to you. Read Chapters 5-10 to understand the actual DigiEquine implementation. Use Chapters 11-14 for validation, interview preparation, and your longer-term learning plan.

| Study Mode | Recommended Chapters | Goal |
| --- | --- | --- |
| Tonight before an interview | 1, 3, 4, 7, 10, 13 | Explain architecture, RTOS primitives, interrupts, faults, and trade-offs. |
| First week of RTOS study | 1-8 | Understand scheduling, communication, synchronization, drivers, and data flow. |
| Professional growth | 9-14 | Learn validation, safety, security, debugging, testing, and release discipline. |

## Contents

1. Firmware Engineering Mindset and Requirements

2. Hardware and System Context

3. RTOS Fundamentals for Beginners

4. Choosing the Correct RTOS Primitive

5. Layered Firmware Architecture

6. Building the Firmware Step by Step

7. Task Design, Scheduling, and Timing

8. Interrupt Design and Deferred Processing

9. Drivers, Services, and Application Policy

10. Detailed Runtime Flow and app_core.c

11. Offline Store-and-Forward Google Drive Upload

12. Reliability, Fault Handling, and Data Integrity

13. Testing, Debugging, CI, and Production Readiness

14. Technical Interview Preparation

15. Roadmap to Experienced RTOS Firmware Developer

Appendix A. Glossary

Appendix B. Review Checklists

# 1. Firmware Engineering Mindset and Requirements

Professional firmware development begins before writing code. The engineer first converts an informal product idea into measurable requirements, identifies timing and safety constraints, maps hardware resources, and defines evidence needed to prove the result works.

## 1.1 Product Goal

DigiEquine is an ESP32-S3-based equine monitoring device. It collects synchronized optical PPG measurements from two MAX30101 sensors, observes an IMU, stores recording sessions on a microSD card, reports status, and accepts commands from an external interface.

## 1.2 Functional Requirements

Start and stop a recording session through a typed command.

Collect fresh paired samples from two PPG sensors.

Timestamp samples using a monotonic microsecond clock.

Write samples to a session file without blocking acquisition.

Run acquisition windows from a 20-second hardware timer schedule.

Report battery, sensor, storage, recording, and fault status.

Stop safely when data integrity cannot be guaranteed.

## 1.3 Non-Functional Requirements

Determinism: acquisition should meet its timing deadline under normal load.

Reliability: sensor, queue, and storage failures must be visible and handled.

Maintainability: board pins, device registers, services, and application policy must be separated.

Testability: pure logic and architecture contracts should be testable without hardware.

Observability: status counters, fault masks, logs, stack margins, and core dumps should support diagnosis.

Security: production communication must use authenticated commands, protected credentials, verified TLS, and signed updates.

Interview principle
Do not say that firmware is production-ready because it looks organized. Production readiness requires measured timing, hardware tests, fault-injection results, security configuration, traceability, and release evidence.

## 1.4 Why the Original Single-File Approach Was Risky

The original Arduino sketch combined hardware configuration, register behavior, BLE commands, WiFi upload, data acquisition, storage, LEDs, and product state. That made ownership unclear. Multiple contexts could access shared pointers and peripherals, and slow operations could interfere with acquisition.

| Risk | Example | Professional Response |
| --- | --- | --- |
| Unclear ownership | Two tasks access SD or a shared buffer | Assign one owning task and communicate through a queue. |
| Timing interference | Serial or SD blocks sensor FIFO draining | Keep acquisition high priority and move slow work to services/tasks. |
| Hidden data corruption | A stale sample is reused for the other sensor | Pair only fresh samples and track sequence/overflow counters. |
| Unsafe callback work | BLE callback performs long operations | Parse, validate, enqueue a command, then return. |
| Unreviewable board assumptions | Pins appear in several source files | Centralize pins and electrical assumptions in board_config.h. |

# 2. Hardware and System Context

An experienced firmware engineer understands the hardware clock tree, buses, interrupt lines, power domains, memory, and peripherals before defining tasks. Software architecture cannot compensate for incorrect electrical assumptions.

## 2.1 Hardware Map

| Device | Interface | Role | Firmware Owner |
| --- | --- | --- | --- |
| ESP32-S3 | Dual-core MCU | Runs ESP-IDF and FreeRTOS | Application/BSP |
| PCA9546A | I2C mux at 0x70 | Selects one downstream sensor channel | pca9546a driver |
| MAX30101 #1 | I2C mux channel 0 | PPG channel 1 | max30101 driver |
| MAX30101 #2 | I2C mux channel 1 | PPG channel 2 | max30101 driver |
| BMI270 | I2C mux channel 2 | Motion sensing | bmi270 driver |
| microSD | SPI/FATFS | Persistent session storage | storage_service |
| Battery divider | ADC | Battery estimate | BSP |
| LEDs | GPIO | Local status | BSP/supervisor |

## 2.2 Centralized Board Configuration

All PCB-dependent values belong in board_config.h. This creates a review point between firmware and electrical engineering. A board revision should not require searching the application for GPIO numbers.

#define BOARD_I2C_FREQUENCY_HZ      400000U

#define BOARD_MUX_CHANNEL_PPG_1     0U

#define BOARD_MUX_CHANNEL_PPG_2     1U

#define BOARD_SD_CS_GPIO            GPIO_NUM_44

Hardware assumption
The SD SPI pins and LED active level remain provisional until confirmed against the released schematic. Professional engineering marks assumptions clearly instead of silently treating them as facts.

## 2.3 Clock and Timing Configuration

| Clock/Timer | Configuration | Reason |
| --- | --- | --- |
| CPU | 240 MHz, dynamic frequency scaling disabled | Predictable execution timing during development and characterization. |
| FreeRTOS tick | 1000 Hz | One tick per millisecond for task delays and timeouts. |
| GPTimer | 1 MHz | One counter unit per microsecond; simple 20-second alarm calculation. |
| I2C | 400 kHz | Improves FIFO-drain throughput while remaining standard fast-mode. |
| Monotonic timestamp | esp_timer_get_time() | Duration and ordering are independent of wall-clock changes. |

## 2.4 Timing Mathematics

The GPTimer resolution is 1,000,000 counts per second. A 20-second alarm therefore uses 20,000,000 counts. The code expresses this as APP_COLLECTION_PERIOD_MS multiplied by 1,000 counts per millisecond.

.resolution_hz = 1000000U

.alarm_count = APP_COLLECTION_PERIOD_MS * 1000ULL

# 3. RTOS Fundamentals for Beginners

A Real-Time Operating System is not simply an operating system that runs fast. It is a scheduling and synchronization framework designed to provide bounded, understandable response behavior. Real-time means correctness depends on both the result and when the result is produced.

## 3.1 Bare-Metal Superloop vs RTOS

| Approach | Strength | Weakness | Use When |
| --- | --- | --- | --- |
| Superloop | Simple and small | One slow operation delays everything | Small systems with few cooperative activities. |
| RTOS | Independent blocking tasks, priorities, synchronization | More design complexity and concurrency risks | Multiple activities with different deadlines and latency. |

## 3.2 Task

A task is an independently scheduled function with its own stack and execution state. A task normally contains an infinite loop. It should spend most of its life blocked, waiting for an event, queue item, notification, or timeout.

static void sensor_task(void *context) {

for (;;) {

ulTaskNotifyTake(pdTRUE, portMAX_DELAY);

/* Perform a released acquisition window. */

}

}

Key beginner insight
A blocked task does not consume CPU. Blocking is healthy. Continuously polling a flag in a tight loop wastes CPU, increases power, and can starve lower-priority tasks.

## 3.3 Scheduler and Context Switch

The scheduler selects the highest-priority task that is ready to run. A context switch saves the current task's CPU registers and restores another task's saved registers. Context switching has overhead, so tasks should be meaningful units of work rather than tiny fragments.

## 3.4 Task States

| State | Meaning | DigiEquine Example |
| --- | --- | --- |
| Running | Currently executing on a CPU core | Sensor task reads a batch. |
| Ready | Can run but waits for a CPU | Supervisor ready while sensor task has higher priority. |
| Blocked | Waiting for time/event/data | Storage waits on sample queue; sensor waits for notification. |
| Suspended | Explicitly prevented from scheduling | Not used; event-driven blocking is preferred. |

## 3.5 Priority

Priority represents urgency caused by a deadline, not business importance. Sensor FIFO draining has a tighter deadline than writing a status message, so the sensor task has a higher priority. Giving every task a high priority destroys the meaning of priority.

## 3.6 Preemption

In a preemptive RTOS, a newly ready higher-priority task can interrupt a lower-priority task. The GPTimer ISR wakes the high-priority sensor task; FreeRTOS may switch to it immediately after the ISR returns.

## 3.7 Tick, Delay, Timeout, and Monotonic Time

Tick: periodic kernel timing interrupt used for scheduling delays and timeouts.

vTaskDelay(): blocks a task for at least a specified tick duration.

Timeout: prevents waiting forever when a resource or response may fail.

esp_timer_get_time(): microsecond monotonic time used for acquisition deadlines and sample timestamps.

Wall-clock time: calendar time used for human filenames; it must not be used for duration measurement because it can jump.

## 3.8 Stack and Heap

Each task has a private stack containing local variables, return addresses, and call frames. Stack overflow can corrupt unrelated state, so V3 enables stack-overflow checking and reports high-water marks. Heap is shared dynamic memory. Professional embedded systems often allocate RTOS objects at startup and avoid runtime allocation during critical operation.

# 4. Choosing the Correct RTOS Primitive

The primitive should express the ownership and communication rule. Many firmware bugs come from using a global flag where a queue, notification, mutex, or event group was required.

| Primitive | Question It Answers | V3 Use |
| --- | --- | --- |
| Queue | What data or command must be consumed exactly once? | Sample batches and commands. |
| Task notification | Which one task should be woken quickly? | GPTimer ISR wakes sensor task. |
| Event group | Which persistent state must many tasks observe? | Recording and fatal-fault bits. |
| Mutex | Who may enter this shared critical resource now? | Protects the status snapshot. |
| Critical section/spinlock | Which very short state update must be atomic across cores? | State and fault-mask update. |
| Timer | When should an event be released? | 20-second periodic acquisition scheduling. |

## 4.1 Queues

A queue copies typed items between tasks. It provides synchronization and buffering. The sample queue decouples fast sensor acquisition from variable SD latency. It is bounded so overload is visible rather than consuming memory indefinitely.

s_sample_queue = xQueueCreate(APP_SAMPLE_QUEUE_LENGTH,

sizeof(app_sample_batch_t));

xQueueSend(s_sample_queue, &batch, APP_STORAGE_QUEUE_TIMEOUT);

xQueueReceive(s_sample_queue, &batch, pdMS_TO_TICKS(100));

Backpressure
When a producer is faster than a consumer, the queue fills. V3 treats a full sample queue as a data-integrity fault, increments a counter, and stops recording. This is stricter and more honest than silently losing unknown data.

## 4.2 Task Notifications

A direct-to-task notification is a lightweight signal stored inside a task control block. It is ideal when one ISR or task needs to wake one known task. It is faster and smaller than creating a semaphore solely for wakeup.

## 4.3 Event Groups

An event group is a set of bits representing persistent conditions. Multiple tasks can observe APP_EVENT_RECORDING. Event groups are not appropriate for sample data because a bit does not carry payload and repeated events can collapse into one state.

## 4.4 Mutex and Priority Inheritance

A mutex protects a resource from simultaneous access. FreeRTOS mutexes support priority inheritance: if a high-priority task waits for a mutex held by a lower-priority task, the lower-priority task can temporarily inherit the higher priority so it releases the mutex sooner.

Best mutex strategy
The best way to avoid priority inversion is often to avoid sharing the peripheral. V3 gives the active file to the storage task and sensors to the sensor task, so broad peripheral mutexes are unnecessary.

## 4.5 Critical Sections

A critical section briefly prevents concurrent modification. On the dual-core ESP32-S3, the portMUX spinlock coordinates access across cores. Critical sections must be extremely short. Never perform logging, blocking calls, I2C, or file I/O inside them.

## 4.6 Semaphores

A binary semaphore represents an available event/resource; a counting semaphore represents multiple identical resources/events. V3 does not use them for the timer wakeup because a task notification is a better one-to-one signal. An experienced engineer can explain why a primitive was not chosen.

# 5. Layered Firmware Architecture

Layering separates policy from mechanism. Policy answers what the product should do; mechanism answers how a device or peripheral performs an operation.

| Layer | Responsibility | Examples |
| --- | --- | --- |
| Application | Product state, commands, scheduling, fault policy | app_core |
| Services | Application-facing capabilities | sensor_service, storage_service, comm_service |
| Device drivers | Register-level device protocols | max30101, bmi270, pca9546a |
| BSP | Board peripheral primitives | GPIO, ADC, I2C transport, power, LEDs |
| Board configuration | PCB-specific constants and assumptions | board_config.h |
| ESP-IDF/HAL | Vendor drivers, FreeRTOS, FATFS | gptimer, I2C, SPI, ADC, NVS |
| Hardware | Physical electronics | ESP32-S3, sensors, SD, LEDs |

## 5.1 Dependency Direction

Dependencies point downward. app_core may call a service; a service may call a driver; a driver may call the BSP. A device driver must not call app_core. This prevents circular dependencies and makes low-level code reusable.

app_core

-> sensor_service

-> max30101 / bmi270

-> pca9546a

-> bsp

-> ESP-IDF driver

## 5.2 Single-Owner Resources

Sensor task owns coordinated sensor acquisition.

Storage task owns the open CSV file.

Supervisor owns product-state transitions and commands.

Communication task owns the input transport.

Device drivers own register protocols; board_config owns pins.

## 5.3 Why Drivers and Services Are Different

The MAX30101 driver knows register addresses, FIFO pointers, device identity, modes, and overflow counters. The sensor service knows that the product needs two fresh paired samples with one timestamp. If the sensor model changes, product policy can remain stable while the driver changes.

# 6. Building the Firmware Step by Step

## Step 1: Inventory hardware

List every peripheral, address, GPIO, clock, bus, interrupt, power domain, and provisional assumption. Capture board-specific values in board_config.h.

## Step 2: Define measurable requirements

Specify sample rate, acquisition window, allowed loss, storage behavior, response times, fault policy, and validation evidence.

## Step 3: Identify concurrency

Separate activities with different blocking behavior or deadlines: acquisition, storage, commands, health reporting.

## Step 4: Assign ownership

Give each mutable resource one owner. Avoid sharing an open file or sensor bus behavior across arbitrary tasks.

## Step 5: Select RTOS primitives

Use queues for payload transfer, notifications for one-task wakeup, events for persistent state, mutexes for short shared snapshots.

## Step 6: Build bottom-up

Implement board configuration and BSP, then device drivers, services, and finally application orchestration.

## Step 7: Add observability

Expose fault masks, counters, status, stack high-water marks, logs, and core dumps.

## Step 8: Add failure policy

Decide whether each failure retries, degrades, stops recording, or restarts.

## Step 9: Verify incrementally

Run formatting, architecture checks, unit tests, builds, hardware tests, fault injection, and soak testing.

## Step 10: Release with evidence

Record source revision, binaries, configuration, test results, known limitations, and approvals.

Why bottom-up?
Higher layers should depend on stable lower-layer contracts. Writing application logic directly against registers creates rework and makes testing difficult.

# 7. Task Design, Scheduling, and Timing

| Task | Priority/Core | Blocks On | Deadline/Reason |
| --- | --- | --- | --- |
| Sensor | 8 / core 1 | Task notification; sensor availability | Highest application urgency because FIFOs can overflow. |
| Storage | 5 / core 0 | Sample queue | Must keep average throughput above producer rate. |
| Supervisor | 4 / core 0 | Command queue timeout | Commands/status are important but less time-critical. |
| Communication | 3 / core 0 | UART input | External interface can tolerate more latency. |

## 7.1 Why the Sensor Task Has Highest Priority

The sensor FIFO is finite. If software fails to drain it before it fills, samples are lost. Storage and communication latency can vary, but they must not prevent FIFO draining. Therefore acquisition has the tightest deadline and the highest application priority.

## 7.2 Why Core Affinity Is Used

Pinning acquisition to core 1 reduces interference from service work on core 0 and simplifies timing characterization. Core affinity is not automatically better; it is a deliberate trade-off. The final decision should be supported by measured latency and CPU utilization.

## 7.3 Period and Window

APP_COLLECTION_PERIOD_MS determines how often the timer releases a collection cycle. APP_COLLECTION_WINDOW_MS determines how long sensors remain active in each cycle. When both are 20 seconds, collection windows are consecutive. A 40-second period and 20-second window creates 20 seconds collecting followed by 20 seconds idle.

## 7.4 Worst-Case Reasoning

Measure worst-case sensor batch read time, not only average time.

Measure worst-case SD write and sync latency.

Ensure the queue absorbs expected temporary storage stalls.

Define the maximum acceptable dropped-batch and FIFO-overflow count.

Measure task CPU time and stack high-water marks during worst-case communication and storage activity.

## 7.5 Rate-Monotonic Intuition

For periodic tasks, shorter periods generally imply higher urgency. Real systems also include blocking and shared resources, so priority assignment must consider deadlines, execution time, blocking time, and interrupt load rather than relying only on period.

# 8. Interrupt Design and Deferred Processing

An interrupt service routine responds to hardware asynchronously. Interrupts can preempt tasks, so they must be bounded and extremely short. The standard professional pattern is: acknowledge the source, capture minimal information, notify a task, request a context switch if needed, and return.

## 8.1 V3 GPTimer Interrupt

static bool IRAM_ATTR collection_timer_alarm_cb(...) {

BaseType_t higher_priority_task_woken = pdFALSE;

vTaskNotifyGiveFromISR(s_sensor_task_handle,

&higher_priority_task_woken);

return higher_priority_task_woken == pdTRUE;

}

The callback does not collect data. It releases the sensor task. This is called deferred interrupt processing: the ISR handles urgency, while a normal task handles complex work.

## 8.2 Why IRAM_ATTR?

IRAM_ATTR requests placement in instruction RAM. On ESP32 systems, flash can temporarily be unavailable during certain operations. ISR code and anything it calls must obey ESP-IDF's interrupt-safety rules when configured for IRAM-safe operation.

## 8.3 Forbidden ISR Operations

Blocking API calls or waiting for a mutex.

I2C, SPI, SD, file, or network transactions.

Dynamic allocation or freeing memory.

Heavy logging or string formatting.

Long loops or complex signal processing.

Calling APIs that are not documented as ISR-safe.

## 8.4 Future Sensor Interrupts

The MAX30101 FIFO is currently polled because V2 did not identify sensor interrupt GPIOs. Once the schematic confirms them, GPIO ISRs should notify the sensor task when FIFO data or almost-full conditions occur. The driver/service/application separation allows this change without rewriting storage or command logic.

# 9. Drivers, Services, and Application Policy

## 9.1 Board Configuration and BSP

board_config.h is the authoritative PCB mapping. The BSP configures GPIO, ADC, and I2C transport and exposes board primitives. It should not contain product policy such as when recording starts.

## 9.2 PCA9546A Driver

The mux driver selects one I2C downstream channel. It hides the selection byte from services. Because both PPG sensors share the same address, selecting the correct mux channel before every operation is essential.

## 9.3 MAX30101 Driver

Probes the part ID before use.

Resets FIFO pointers and overflow counter on start.

Configures sample and LED registers.

Checks whether a fresh sample is available.

Reads an 18-bit FIFO sample.

Tracks FIFO overflow count.

Shuts down the sensor when a collection window ends.

## 9.4 Sensor Service

The service coordinates two device instances. A row is produced only when both sensors have fresh data. It assigns a sequence number and monotonic timestamp. This is product-level acquisition policy, not register-level behavior.

## 9.5 Storage Service

The storage service owns the active file and hides FATFS details. It writes a versioned header, sequence/timestamp/data rows, periodically synchronizes the file, and adds a row-count and checksum footer when the session closes.

Power-loss trade-off
Calling fsync after every row improves durability but creates large latency and wear. V3 synchronizes every 16 batches as a compromise. The correct interval must be chosen from product loss tolerance and measured SD latency.

## 9.6 Communication Service

The baseline communication service converts UART text into typed commands and publishes status. A future BLE adapter should perform validation and enqueue the same app_command_t type. BLE callbacks must not directly manipulate sensors, storage, or application state.

# 10. Detailed Runtime Flow and app_core.c

app_core.c is the orchestration layer. It creates RTOS objects, owns system state transitions, coordinates tasks, and applies fault policy. It intentionally does not know MAX30101 registers or CSV implementation details.

## 10.1 Startup Flow

Initialize NVS; erase and reinitialize only for known recoverable NVS format/full conditions.

Initialize BSP hardware primitives.

Create event group, sample queue, command queue, and status mutex.

Create and enable the GPTimer, but do not start it.

Initialize sensor and storage services.

Latch a startup fault if a required subsystem is unavailable.

Initialize communications.

Create sensor, storage, and supervisor tasks.

## 10.2 START Command Flow

Communication task

-> command queue

-> supervisor_task()

-> handle_command(START)

-> set APP_EVENT_RECORDING

-> notify sensor task immediately

-> start periodic GPTimer

## 10.3 Sensor-to-Storage Flow

GPTimer ISR / START command

-> task notification

-> sensor_task

-> sensor_service_read_batch

-> bounded sample queue

-> storage_task

-> storage_service_write_batch

-> FATFS / microSD

## 10.4 Function Responsibilities

| Function | Responsibility | RTOS Concept |
| --- | --- | --- |
| latch_fault | Atomically record fault, stop recording/timer, publish fatal state | Critical section + event group |
| collection_timer_alarm_cb | Release sensor task every period | ISR + task notification |
| collection_timer_init/start/stop | Manage GPTimer lifecycle | Hardware timer |
| status_update_health | Refresh protected status snapshot | Mutex |
| sensor_task | Run acquisition windows and publish batches | High-priority task + queue |
| storage_task | Own session file and consume batches | Single-owner task + queue |
| handle_command | Serialize application state transitions | Command pattern |
| supervisor_task | Process commands and publish health/status | Control task |
| app_core_start | Construct system and start tasks | Initialization |

## 10.5 Fault Flow

If sensor start/read, storage start/write, or queue transfer fails, app_core increments observable counters and calls latch_fault(). The fault mask preserves all fault categories. Recording and the GPTimer stop because continuing would produce incomplete or misleading research data.

## 10.6 State Machine

| State | Entry | Exit |
| --- | --- | --- |
| BOOT | Reset and initialization | IDLE if healthy; FAULT if required startup fails. |
| IDLE | Healthy initialization or STOP | RECORDING on START; FAULT on failure. |
| RECORDING | Accepted START command | IDLE on STOP; FAULT on integrity failure. |
| FAULT | Latched fatal fault | Currently requires restart/recovery policy. |

# 11. Offline Store-and-Forward Google Drive Upload

WiFi and Google Drive are unreliable external dependencies. A mare may leave coverage for hours, weak connectivity can make transfers slow, and an upload may fail after partially transferring data. Therefore acquisition must always store locally first. Upload is asynchronous, resumable, persistent across reboots, and lower priority than collection and storage.

| Task | Priority | Responsibility | Failure Result |
| --- | --- | --- | --- |
| Sensor | 8 | Drain FIFOs and create paired samples | Permanent sample loss |
| Storage | 6 | Persist and finalize session files | Local data loss/corruption |
| Supervisor | 4 | Commands, health, and faults | Slower control response |
| Communication | 3 | BLE/status interaction | Slower UI response |
| Upload | 2 | Transfer completed files when possible | Backlog grows safely |

Collection-first rule
Sensor collection and local storage must never wait for WiFi, Google Drive, or upload completion.

## 11.1 Store-and-Forward Pipeline

Sensors -> Sensor Task -> Sample Queue -> Storage Task -> SD .ready files

|

v

low-priority Upload Task -> WiFi -> Drive

The active recording file is never uploaded. Storage closes, synchronizes, and validates a session before atomically marking it ready. If the mare leaves coverage, completed ready files accumulate on SD. When connectivity returns, the upload task processes the oldest files while new collection continues.

## 11.2 Persistent File Lifecycle

| State | Meaning | Policy |
| --- | --- | --- |
| .part | Active/incomplete recording | Storage writes; upload ignores. |
| .ready | Closed and verified locally | Eligible for oldest-first upload. |
| .uploading | Transfer in progress | Persist resumable session and confirmed offset. |
| .uploaded | Drive success verified | Retain or delete under retention policy. |

## 11.3 Weak WiFi and Reconnection

Connectivity loss pauses upload without changing recording state.

Upload progress and retry metadata are persisted.

Reconnect attempts use exponential backoff and jitter.

When coverage returns, pending files resume oldest-first.

Backlog count and remaining SD capacity are published in status.

## 11.4 Preserve SD Ownership

The safest design keeps the storage task as the only filesystem owner. The upload task requests small chunks through a storage-request queue. Recording writes are always serviced before background upload reads, preventing upload directory scans or reads from delaying critical sample persistence.

if (sample_batch_available()) {

write_recording_batch();       /* critical path */

} else if (upload_chunk_requested()) {

read_upload_chunk();           /* best-effort background */

}

## 11.5 Resumable, Idempotent, and Secure Upload

Give every session a globally unique ID used as an idempotency key.

Persist resumable upload session information and last confirmed byte offset.

Retrying after lost acknowledgment must not create duplicate Drive files.

Accept only verified successful responses and record the Drive file ID.

Verify TLS certificates and protect credentials; never use insecure TLS.

Use bounded chunks so upload cannot monopolize memory, CPU, or SD access.

## 11.6 Storage-Full Policy

Delete oldest confirmed uploaded files first when retention policy permits.

Never automatically delete pending, uploading, active, or unverified files.

Warn early when free space is low and publish backlog size.

Stop recording safely before overwriting unuploaded research data.

# 12. Reliability, Fault Handling, and Data Integrity

## 11.1 Fault Taxonomy

| Fault | Meaning | Current Policy |
| --- | --- | --- |
| SENSOR_START | Required sensor could not start | Latch fault and stop recording. |
| SENSOR_RUNTIME | Sensor read failed during acquisition | Latch fault and stop recording. |
| STORAGE_START | Session file could not open | Latch fault and stop recording. |
| STORAGE_RUNTIME | Batch write/sync failed | Latch fault and stop recording. |
| QUEUE_OVERFLOW | Storage could not keep up | Count loss, latch fault, stop recording. |

## 11.2 Why Stop on Data-Integrity Fault?

For a research instrument, silent partial data may be worse than no data. Continuing after unknown loss could make downstream analysis appear valid. The safe policy is to stop, preserve the fault evidence, and require explicit recovery.

## 11.3 Sequence Numbers and Timestamps

Sequence numbers expose missing or duplicated records. Monotonic timestamps expose timing gaps and jitter. Together they allow offline validation rather than trusting that every row is continuous.

## 11.4 Checksum and Row Count

The session footer stores the number of rows and an FNV-1a rolling checksum over explicit sample fields. This is not cryptographic authentication, but it helps detect accidental corruption or incomplete processing. Security-sensitive integrity requires a cryptographic MAC or signature.

## 11.5 Watchdog, Stack Checks, and Core Dumps

Task watchdog detects tasks that fail to yield or service expected execution.

Stack-overflow canary detects task stack corruption.

Stack high-water marks show the minimum remaining stack during execution.

Core dumps preserve crash context for postmortem debugging.

## 11.6 Recovery Policy Questions

Is the fault transient, recoverable, degraded, or fatal?

How many retries are allowed and with what backoff?

Can the system continue without compromising data meaning?

What evidence is recorded before restart?

Who authorizes destructive recovery such as deleting data?

# 13. Testing, Debugging, CI, and Production Readiness

## 12.1 Test Pyramid

| Level | Purpose | Examples |
| --- | --- | --- |
| Static checks | Catch structure/style problems quickly | clang-format, compiler warnings, architecture checker. |
| Unit tests | Verify pure logic in isolation | FNV-1a known vectors, parsers, state transitions. |
| Integration tests | Verify component contracts | Mock sensor driver + sensor service; storage lifecycle. |
| Hardware-in-loop | Verify actual timing/electrical behavior | Sensor disconnect, SD removal, FIFO overflow. |
| Soak/stress | Find rare timing and resource failures | 24-hour recording with communication traffic. |

## 12.2 Architecture Contract Check

tools/check_architecture.py enforces key decisions: board pins remain centralized, services do not contain device register definitions, and the timer ISR contains no forbidden heavy operations. This is a lightweight example of turning architecture into an executable rule.

## 12.3 Fault Injection

Disconnect each PPG sensor before and during recording.

Remove, fill, or corrupt the SD card.

Delay storage artificially until the queue fills.

Force NVS version/full conditions.

Generate communication traffic during acquisition.

Power-cycle during file creation, write, sync, and close.

Trigger watchdog and stack-overflow test builds in a controlled environment.

Disconnect/reconnect WiFi while processing a large upload backlog.

Reboot during upload and verify resume without duplicate Drive files.

Fill SD with pending uploads and verify that unuploaded data is never deleted.

## 12.4 Measuring Real-Time Performance

Measure interrupt-to-task wakeup latency.

Measure sensor batch execution time and jitter.

Measure worst-case storage write and fsync latency.

Measure queue depth over time under worst-case load.

Measure FIFO overflow and dropped-batch counters.

Measure CPU utilization and stack high-water marks.

## 12.5 CI and Release Discipline

The GitHub workflow runs architecture checks, formatting checks, and a clean ESP32-S3 build. A professional release additionally archives binaries, source revision, configuration, test evidence, known issues, and approvals. CI passing is necessary but not sufficient for hardware production.

Current honest status
The repository is a production-oriented baseline, not a production-qualified release. Hardware pins, sensor interrupts, BMI270 initialization, target build environment, hardware tests, security, and release evidence remain open.

# 14. Technical Interview Preparation

## 13.1 Two-Minute Project Explanation

I migrated a single-file Arduino firmware into a layered ESP-IDF architecture. Board-specific values are centralized, register-level device protocols are isolated in drivers, and services expose application-facing operations. A high-priority sensor task collects fresh paired PPG samples and sends bounded batches to a single-owner storage task. A supervisor serializes commands and publishes health. A 20-second GPTimer ISR only notifies the sensor task, demonstrating deferred interrupt processing. Failures that threaten data integrity latch a fault and safely stop recording. The design includes sequence numbers, overflow counters, checksums, watchdogs, formatting checks, CI, and a documented validation roadmap.

## 13.2 Common Questions and Strong Answers

### Why use RTOS?

The product has activities with different deadlines and blocking behavior. Acquisition must drain finite FIFOs, while storage and communication can block. Tasks, priorities, queues, and notifications isolate those concerns and make latency measurable.

### Why not perform collection in the timer ISR?

I2C and storage operations are long, may block, and are not ISR-safe. The ISR only wakes a high-priority task; complex work runs in task context.

### Why a queue between sensor and storage?

It transfers ownership of typed batches, decouples timing, provides bounded buffering, and makes backpressure visible.

### Why no SD mutex?

The storage task exclusively owns the open file. Single ownership avoids broad locking and priority inversion.

### Why task notification instead of semaphore?

Only one known task needs waking. A direct notification is lighter and expresses the one-to-one relationship.

### What is priority inversion?

A high-priority task waits for a resource held by a low-priority task while medium-priority work delays the owner. Mutex priority inheritance helps, but avoiding shared ownership is stronger.

### How do you prove real-time behavior?

Measure worst-case execution, interrupt latency, blocking time, queue depth, FIFO overflow, dropped data, CPU utilization, and stack margins under worst-case load.

### Is it production-ready?

It is production-oriented but requires target build success, schematic review, hardware-in-loop validation, security configuration, soak testing, and release evidence.

### How do you handle weak WiFi?

Data is stored locally first. A low-priority store-and-forward upload task resumes completed files when connectivity returns; WiFi never blocks acquisition.

## 13.3 Questions You Should Ask the Interviewer

What are the hardest real-time deadlines in your product, and how are they measured?

How are hardware-in-loop tests and fault injection performed?

What coding standard, static analysis, and CI gates are required?

How does the team handle device-driver ownership and board revisions?

What is the firmware release, OTA rollback, and field-debugging strategy?

## 13.4 Avoid These Weak Answers

| Weak Statement | Better Statement |
| --- | --- |
| RTOS makes firmware faster. | RTOS structures concurrency and response priorities; it also adds overhead and must be designed carefully. |
| The ISR collects every 20 seconds. | The GPTimer ISR releases a task; acquisition happens safely in task context. |
| There are no race conditions. | Ownership and synchronization reduce races; validation and analysis are still required. |
| It is industry-ready. | It is production-oriented with explicit remaining qualification gates. |

# 15. Roadmap to Experienced RTOS Firmware Developer

Experience comes from repeated design, measurement, failure analysis, and hardware debugging. Memorizing API names is not enough. Use this project as a laboratory and deliberately add evidence.

## 14.1 Four-Week Learning Plan

| Week | Focus | Practice |
| --- | --- | --- |
| 1 | FreeRTOS fundamentals | Create tasks, queues, notifications, event groups, mutexes; observe task states. |
| 2 | Drivers and interrupts | Implement GPIO interrupt-to-task notification; measure latency; build mockable driver APIs. |
| 3 | Reliability and timing | Inject delays/faults, measure queue depth and worst-case time, analyze stack margins. |
| 4 | Production workflow | Add tests, CI, versioning, release notes, signed OTA design, and hardware validation plan. |

## 14.2 Practical Exercises on V3

Add confirmed MAX30101 interrupt GPIOs and replace FIFO polling with task notifications.

Create a mocked MAX30101 transport and unit-test overflow and sample availability behavior.

Add a storage fault-injection mode that delays writes and prove queue-overflow handling.

Add task runtime statistics and produce a CPU-utilization report.

Implement a nonfatal/degraded fault category and document recovery transitions.

Implement authenticated BLE command parsing that only enqueues typed commands.

Implement persistent file lifecycle and resumable, idempotent Google Drive upload.

Add firmware version/build ID to every session header and status report.

Design and test signed OTA with rollback and power-loss recovery.

## 14.3 Topics to Master Next

Rate-monotonic scheduling and response-time analysis.

Memory barriers, atomics, cache behavior, and multicore concurrency.

DMA, zero-copy buffers, ring buffers, and interrupt latency.

Bootloaders, secure boot, flash encryption, OTA, and rollback.

MISRA C concepts, static analysis, defensive coding, and traceability.

Oscilloscope/logic-analyzer timing validation and hardware debugging.

Low-power states, wake sources, clock scaling, and power budgeting.

Safety and regulated-development concepts appropriate to the target industry.

Professional habit
For every design decision, practice answering: What requirement caused it? What alternatives existed? What failure does it prevent? What trade-off does it introduce? What evidence proves it works?

# Appendix A. RTOS and Firmware Glossary

| Term | Meaning |
| --- | --- |
| Atomic | An operation observed as indivisible; no other context sees a partial update. |
| Backpressure | A condition where a consumer cannot keep up with a producer. |
| BSP | Board Support Package; board-specific peripheral primitives. |
| Context switch | Saving one task's CPU context and restoring another's. |
| Critical section | A very short region protected from concurrent access. |
| Deadline | Latest time by which a result must be produced. |
| Determinism | Ability to bound and understand timing behavior. |
| Event group | RTOS object storing shared persistent state bits. |
| FIFO | First-In, First-Out buffer. |
| HAL | Hardware Abstraction Layer supplied by vendor/framework. |
| ISR | Interrupt Service Routine; code executed in response to hardware interrupt. |
| Jitter | Variation in timing from one execution/event to another. |
| Latency | Time between an event and the required response. |
| Monotonic time | Time source that only moves forward; appropriate for durations. |
| Mutex | Mutual-exclusion object protecting a shared resource. |
| Priority inversion | High-priority task waits indirectly behind lower-priority work. |
| Queue | RTOS object that transfers typed items between contexts. |
| Race condition | Behavior depends incorrectly on unpredictable execution ordering. |
| Semaphore | Synchronization object representing events or resource counts. |
| Task notification | Lightweight signal sent directly to one FreeRTOS task. |
| WCET | Worst-Case Execution Time. |

# Appendix B. Design Review Checklist

Requirements contain measurable timing, loss, recovery, and security criteria.

Board configuration matches the released schematic.

Each peripheral and mutable resource has a documented owner.

Task priorities are based on deadlines and measured blocking.

Every queue is bounded and has an overload policy.

ISRs call only ISR-safe, bounded operations.

Critical sections are short and contain no blocking work.

Faults are classified and recovery behavior is documented.

Data contains sequence/timing/integrity evidence appropriate to its use.

Unit, integration, hardware, fault-injection, and soak tests have evidence.

Security and update mechanisms match product risk.

Release artifacts are traceable to source and configuration.

# Appendix C. Final Study Summary

The strongest mental model is ownership plus deadlines. Decide which work has a deadline, give it an appropriate task priority, give each peripheral one owner, transfer data through bounded queues, use notifications for urgent one-task wakeups, use event groups for persistent state, keep ISRs minimal, and stop safely when integrity cannot be guaranteed. Then prove the design with measurements and fault injection.