DIGIEQUINE V3

Step-by-Step Firmware Implementation Manual

System configuration, drivers, RTOS tasks, synchronization, interrupts, scheduling, and code reasoning

How this manual is different
This manual follows the order an experienced firmware engineer uses to construct the system. It explains the objective, code, line-level mechanism, alternative choices, and engineering reason for each major implementation block.

ESP32-S3 | ESP-IDF | FreeRTOS | June 4, 2026

# How an Experienced Firmware Engineer Builds the System

Firmware is not built by creating tasks first. The professional sequence is: understand requirements and hardware, configure the platform, define ownership and timing, implement low-level drivers, compose services, create RTOS communication contracts, implement tasks and interrupts, add fault handling, and finally prove the system with measurement.

Translate the product idea into measurable functional and timing requirements.

Create the board and system configuration before writing application behavior.

Build device drivers that know registers but do not know product policy.

Build services that compose drivers into product capabilities.

Define task ownership, priorities, queues, events, mutexes, and notifications.

Create application orchestration and explicit state/fault transitions.

Validate timing, memory, faults, storage integrity, security, and release evidence.

## Document Map

| Part | Main Question |
| --- | --- |
| 1. Engineering preparation | What must be decided before code? |
| 2. System configuration | How are CPU, RTOS, watchdog, memory, and diagnostics configured? |
| 3. Board/BSP/drivers | How is hardware isolated and configured safely? |
| 4. Services | How are drivers composed into product capabilities? |
| 5. RTOS concepts | How do tasks, threads, queues, mutexes, events, and scheduling differ? |
| 6. Application code | How does app_core.c coordinate the entire system? |
| 7. Validation | How do we prove the design works? |

Important honesty
The code is a production-oriented baseline. Hardware-dependent behavior must still be confirmed by schematic review, a successful target build, instrumentation, fault injection, and hardware-in-loop tests.

# 1. Begin With Requirements, Not APIs

## 1.1 Convert Product Statements Into Engineering Requirements

| Product Statement | Engineering Requirement | Design Consequence |
| --- | --- | --- |
| Collect two PPG channels | Every stored row must contain two fresh samples | Sensor service pairs fresh samples; stale reuse is forbidden. |
| Collect every 20 seconds | A periodic hardware timer releases an acquisition window | GPTimer ISR notifies sensor task. |
| Do not lose research data | Loss must be detectable and recording stops if integrity fails | Bounded queue, sequence numbers, overflow counters, fault latch. |
| Save to SD | Variable-latency storage cannot block acquisition | Dedicated storage task owns file. |
| Show system health | Status is published periodically and protected from races | Supervisor plus status mutex. |

## 1.2 Identify Timing Classes

Hard/firm deadline-like work: drain sensor FIFOs before overflow.

Soft real-time work: status publication and LED updates.

Variable latency work: SD writes, file sync, communication.

Asynchronous work: timer interrupts and external commands.

## 1.3 Assign Resource Ownership

| Resource | Owner | Reason |
| --- | --- | --- |
| PPG acquisition policy | Sensor task | One execution context coordinates two sensors. |
| Open session file | Storage task | Prevents concurrent writes and file corruption. |
| Application state transitions | Supervisor task | Serializes commands and policy decisions. |
| Status snapshot | Shared, protected by mutex | Several contexts update/read a small structure. |
| Fault mask/state | Shared, protected by short critical section | Atomic update across ESP32-S3 cores. |

# 2. System Configuration: Configure the Platform First

sdkconfig.defaults defines reproducible platform behavior. Configuration belongs in version control because CPU frequency, tick rate, watchdogs, stack checks, core dumps, and communication stacks change runtime behavior.

CONFIG_IDF_TARGET_ESP32S3=y

CONFIG_FREERTOS_HZ=1000

CONFIG_ESP_DEFAULT_CPU_FREQ_MHZ_240=y

CONFIG_ESP_DEFAULT_CPU_FREQ_MHZ=240

CONFIG_PM_ENABLE=n

CONFIG_FREERTOS_CHECK_STACKOVERFLOW_CANARY=y

CONFIG_ESP_TASK_WDT_EN=y

CONFIG_ESP_TASK_WDT_TIMEOUT_S=8

CONFIG_ESP_COREDUMP_ENABLE_TO_FLASH=y

CONFIG_ESP_COREDUMP_DATA_FORMAT_ELF=y

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| CONFIG_IDF_TARGET_ESP32S3 | Selects ESP32-S3 headers, linker scripts, peripheral capabilities, and toolchain. | A firmware build must target the released MCU explicitly. |
| CONFIG_FREERTOS_HZ=1000 | Sets the kernel tick to 1 ms. | Provides convenient millisecond delays/timeouts; increases tick overhead compared with lower rates. |
| CPU_FREQ=240 | Runs CPU at a fixed 240 MHz. | Simplifies timing characterization during development. |
| CONFIG_PM_ENABLE=n | Disables dynamic frequency scaling. | Reduces timing variability; trades away power savings. |
| STACKOVERFLOW_CANARY | Adds runtime stack boundary detection. | Task stack corruption is catastrophic and otherwise difficult to diagnose. |
| TASK_WDT / 8 s | Detects tasks that fail to yield for too long. | Finds deadlocks, runaway loops, and blocking bugs. |
| COREDUMP | Stores crash context in flash. | Supports postmortem diagnosis instead of guessing. |

## 2.1 Application Configuration

#define APP_SENSOR_TASK_PRIORITY       8

#define APP_STORAGE_TASK_PRIORITY      5

#define APP_SUPERVISOR_TASK_PRIORITY   4

#define APP_COMM_TASK_PRIORITY         3

#define APP_SAMPLE_QUEUE_LENGTH        12

#define APP_COLLECTION_PERIOD_MS       20000U

#define APP_COLLECTION_WINDOW_MS       20000U

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| Sensor priority 8 | Makes acquisition the most urgent application task. | Finite sensor FIFOs create the tightest deadline. |
| Storage priority 5 | Allows storage to drain batches while yielding to acquisition. | SD latency varies and must not preempt sensor deadlines. |
| Supervisor priority 4 | Processes commands and health below data-path work. | Control responsiveness matters but does not justify interfering with acquisition. |
| Communication priority 3 | Keeps input/output lowest among application tasks. | External interfaces normally tolerate more latency. |
| Queue length 12 | Buffers twelve fixed-size sample batches. | Bounded memory plus short-term absorption of storage stalls. |
| Period/window 20 s | Releases and runs consecutive 20-second windows. | Separates scheduling period from active duration for future duty cycling. |

Experienced-engineer question
Priority values are hypotheses until measured. After hardware tests, inspect FIFO overflow, queue depth, task runtime, latency, and stack margins. Adjust priorities from evidence, not intuition alone.

# 3. Board Configuration and BSP

## 3.1 Centralize PCB Assumptions

#define BOARD_I2C_PORT              I2C_NUM_0

#define BOARD_I2C_SDA_GPIO          GPIO_NUM_5

#define BOARD_I2C_SCL_GPIO          GPIO_NUM_6

#define BOARD_I2C_FREQUENCY_HZ      400000U

#define BOARD_I2C_TIMEOUT_MS        50U

#define BOARD_MUX_CHANNEL_PPG_1     0U

#define BOARD_MUX_CHANNEL_PPG_2     1U

#define BOARD_MUX_CHANNEL_IMU       2U

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| BOARD_I2C_PORT | Selects one ESP32 I2C controller. | Only board configuration should know which controller is wired. |
| SDA/SCL GPIO | Maps physical PCB pins. | A board revision changes one reviewed file, not application code. |
| 400000U | Uses I2C fast-mode at 400 kHz. | Improves FIFO-draining throughput while remaining standard. |
| 50U timeout | Bounds an I2C transaction. | A stuck sensor must not block a real-time task forever. |
| MUX channels | Names physical downstream routing. | Avoids unexplained numeric channel constants in services. |

## 3.2 BSP Objective

The Board Support Package converts board configuration into reusable peripheral primitives. It knows how to configure GPIO, ADC, I2C transport, sensor power, and LEDs. It does not know recording policy or MAX30101 register meanings.

i2c_config_t bus_config = {

.mode = I2C_MODE_MASTER,

.sda_io_num = BOARD_I2C_SDA_GPIO,

.scl_io_num = BOARD_I2C_SCL_GPIO,

.sda_pullup_en = GPIO_PULLUP_ENABLE,

.scl_pullup_en = GPIO_PULLUP_ENABLE,

.master.clk_speed = BOARD_I2C_FREQUENCY_HZ,

};

ESP_RETURN_ON_ERROR(i2c_param_config(BOARD_I2C_PORT, &bus_config), TAG,

"I2C config failed");

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| i2c_config_t | Creates a typed configuration structure. | Structured configuration is clearer and safer than unrelated calls. |
| .mode | Configures ESP32 as bus master. | The ESP32 initiates sensor transactions. |
| .sda/.scl | Applies reviewed board pins. | Keeps board knowledge outside drivers. |
| pullup enable | Enables internal pullups. | Useful during development; external pullups should be confirmed electrically. |
| clk_speed | Uses centralized I2C frequency. | One source of truth. |
| ESP_RETURN_ON_ERROR | Returns immediately and logs context on failure. | Initialization must fail clearly rather than continue partially configured. |

## 3.3 Bounded I2C Transaction

return i2c_master_write_read_device(

BOARD_I2C_PORT, address, &reg, 1, data, length,

pdMS_TO_TICKS(BOARD_I2C_TIMEOUT_MS));

This performs a register-address write followed by a read. The timeout is converted from milliseconds to RTOS ticks. The driver receives success or a specific error, so it can apply policy rather than hanging indefinitely.

# 4. Device Drivers: Register-Level Mechanism

A device driver owns the protocol of one device. It should expose meaningful operations such as probe, start, stop, available, and read. It should not know whether the product is recording a horse or writing CSV.

## 4.1 PCA9546A Driver

esp_err_t pca9546a_select(uint8_t channel) {

return bsp_i2c_mux_select(channel);

}

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| pca9546a_select | Exposes a device-named operation. | Services and drivers should depend on device meaning, not raw BSP naming. |
| return bsp... | Delegates physical I2C transmission to BSP. | Preserves downward dependency direction. |

## 4.2 MAX30101 Driver Instance

typedef struct {

uint8_t mux_channel;

uint8_t last_overflow_value;

uint32_t fifo_overflow_count;

} max30101_t;

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| mux_channel | Identifies which physical sensor instance is selected. | Both devices use the same I2C address, so channel is instance identity. |
| last_overflow_value | Stores previous hardware overflow counter value. | Allows calculation of new overflow events. |
| fifo_overflow_count | Accumulates observed loss. | Makes data-quality problems visible. |

## 4.3 Probe Before Use

ESP_RETURN_ON_ERROR(select_device(device), TAG, "mux select failed");

uint8_t part_id = 0;

ESP_RETURN_ON_ERROR(bsp_i2c_read(DEVICE_ADDRESS, REG_PART_ID,

&part_id, 1), TAG,

"part ID read failed");

return part_id == EXPECTED_PART_ID ? ESP_OK : ESP_ERR_NOT_FOUND;

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| select_device | Routes I2C to the correct sensor. | Required before every operation because addresses are identical. |
| part_id = 0 | Creates deterministic output storage. | Avoids using uninitialized data if debugging. |
| bsp_i2c_read | Reads the documented identity register. | Proves communication and device identity. |
| ternary return | Returns success only for expected hardware. | Prevents configuring the wrong or absent device. |

## 4.4 Start and Configure

write_u8(REG_MODE_CONFIG, 0x40);   /* reset */

vTaskDelay(pdMS_TO_TICKS(20));

write_u8(REG_FIFO_WR_PTR, 0);

write_u8(REG_OVF_COUNTER, 0);

write_u8(REG_FIFO_RD_PTR, 0);

write_u8(REG_FIFO_CONFIG, 0x4F);

write_u8(REG_SPO2_CONFIG, 0x57);

write_u8(REG_LED1_PA, 0x1F);

write_u8(REG_MODE_CONFIG, 0x02);   /* begin sampling */

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| Reset | Places device into known state. | Initialization must not depend on previous power/runtime state. |
| vTaskDelay 20 ms | Blocks task while hardware reset completes. | Blocking lets other tasks run; busy waiting would waste CPU. |
| FIFO pointers = 0 | Clears old FIFO state. | A new recording must not include previous samples. |
| FIFO config 0x4F | Configures averaging/threshold behavior. | Register values belong in device driver and require datasheet review. |
| SPO2 config 0x57 | Configures ADC range, sample rate, and pulse width. | Timing/data quality depend on correct register fields. |
| LED current | Sets optical drive amplitude. | Must be characterized for signal quality, power, and safety. |
| Mode 0x02 | Starts sampling. | Start is last, after all configuration is valid. |

Driver review rule
Every register constant must be traceable to a datasheet revision and requirement. Magic values can remain compact in code, but their bit fields and rationale must be documented and tested.

# 5. Service Layer: Compose Drivers Into Product Behavior

## 5.1 Sensor Service Instances

static max30101_t s_ppg_1 = {

.mux_channel = BOARD_MUX_CHANNEL_PPG_1

};

static max30101_t s_ppg_2 = {

.mux_channel = BOARD_MUX_CHANNEL_PPG_2

};

static uint32_t s_sequence;

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| s_ppg_1 / s_ppg_2 | Creates two independent driver instances. | One driver implementation supports multiple physical devices. |
| Board mux constants | Binds instances to reviewed hardware routing. | Service does not use unexplained channel numbers. |
| s_sequence | Counts produced paired samples. | Offline tools can detect missing or duplicate rows. |

## 5.2 Pair Only Fresh Samples

max30101_sample_available(&s_ppg_1, &first_available);

max30101_sample_available(&s_ppg_2, &second_available);

if (!first_available || !second_available) {

vTaskDelay(pdMS_TO_TICKS(1));

continue;

}

max30101_read_sample(&s_ppg_1, &sample->ppg_1);

max30101_read_sample(&s_ppg_2, &sample->ppg_2);

sample->timestamp_us = (uint64_t)esp_timer_get_time();

sample->sequence = s_sequence++;

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| Availability checks | Confirms each FIFO has a fresh sample. | Prevents pairing a new value with a stale previous value. |
| If unavailable | Blocks for 1 ms then retries. | Avoids a CPU-burning polling loop. |
| Read both | Consumes one sample from each sensor. | Creates a defined paired-row policy. |
| Monotonic timestamp | Records actual acquisition time. | Does not assume loop timing or wall clock. |
| Sequence++ | Creates ordered identity. | Supports loss and duplication detection. |

## 5.3 Batch Deadline

const int64_t deadline_us = esp_timer_get_time() + 100000;

while (batch->count < APP_SAMPLE_BATCH_SIZE &&

esp_timer_get_time() < deadline_us) {

...

}

The service attempts to fill a fixed batch but returns after 100 ms even if it cannot. The deadline prevents a service call from blocking the sensor task forever and allows app_core to observe timeouts/faults.

## 5.4 Storage Service and File Ownership

Only the storage task calls the storage service during recording. The service owns the FILE pointer and session integrity counters. This avoids a shared-file mutex and makes the ownership rule obvious.

if (++s_batches_since_sync >= 16U) {

s_batches_since_sync = 0;

if (fflush(s_session_file) != 0 ||

fsync(fileno(s_session_file)) != 0) {

return ESP_FAIL;

}

}

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| Increment batch counter | Tracks durability interval. | Avoids synchronizing every row. |
| Threshold 16 | Syncs periodically. | Balances power-loss exposure against SD latency/wear. |
| fflush | Moves C library buffer toward filesystem. | Buffered writes are not necessarily durable. |
| fsync | Requests filesystem/device persistence. | Reduces data loss during unexpected power removal. |
| Return ESP_FAIL | Propagates storage failure upward. | app_core can safely stop recording. |

# 6. Deep RTOS Concepts and Comparisons

## 6.1 Tasks vs Threads

In general operating-system terminology, a thread is a schedulable execution path inside a process. FreeRTOS uses the term task for its schedulable execution unit. In this ESP-IDF firmware, FreeRTOS tasks behave similarly to threads: they share the same address space and global variables, while each has its own stack and scheduling state.

| Concept | FreeRTOS Task | Desktop Thread |
| --- | --- | --- |
| Address space | Shared firmware address space | Usually shared within a process |
| Stack | Private task stack | Private thread stack |
| Scheduling | Priority-focused real-time scheduler | General-purpose scheduler |
| Protection | No process isolation between tasks | Process provides isolation from other processes |
| Failure impact | One bad pointer can corrupt whole firmware | Can corrupt process; OS protects other processes |

Interview answer
On FreeRTOS, task is the kernel's term for a thread-like schedulable unit. Tasks share memory, so synchronization and ownership are essential. They are not isolated processes.

## 6.2 Semaphore vs Mutex

| Property | Semaphore | Mutex |
| --- | --- | --- |
| Purpose | Signal event or count resources | Protect one shared resource |
| Ownership | No strict owner; another context may give | Owner takes and must release |
| Priority inheritance | Typically no | Yes in FreeRTOS mutex |
| ISR use | ISR-safe give variants exist | Must not use mutex from ISR |
| V3 choice | Not needed for timer wakeup | Status snapshot protection |

The timer wakeup uses a task notification instead of a binary semaphore because exactly one known task must be released. The status structure uses a mutex because tasks share and update a resource.

## 6.3 Queue vs Event Group vs Notification

| Primitive | Carries Payload? | Persistent? | Best Use | V3 Example |
| --- | --- | --- | --- | --- |
| Queue | Yes | Until consumed | Transfer typed data once | Samples/commands |
| Event group | Bits only | Yes | Broadcast system conditions | Recording/fatal fault |
| Task notification | Small value/count | Until task consumes | Wake one task efficiently | GPTimer to sensor |

## 6.4 Context Switching

A context switch occurs when the scheduler stops one task and runs another. The kernel saves CPU registers, stack pointer, and task state, then restores another task. It may occur because a task blocks, yields, its time slice ends, or a higher-priority task becomes ready.

GPTimer interrupt occurs.

ISR calls vTaskNotifyGiveFromISR().

Sensor task changes from Blocked to Ready.

The callback reports that a higher-priority task was woken.

After ISR exit, scheduler may context-switch immediately to sensor task.

Context-switch trade-off
Context switches improve responsiveness but consume CPU time and disturb caches. Design tasks around meaningful responsibilities and block efficiently instead of creating many tiny tasks.

## 6.5 Scheduling

FreeRTOS normally uses fixed-priority preemptive scheduling. The highest-priority Ready task runs. Equal-priority tasks may time-slice. A blocked high-priority task consumes no CPU, which is why high priority is safe only when the task blocks correctly.

## 6.6 Priority Inversion

Priority inversion occurs when a high-priority task waits for a resource owned by a low-priority task, while medium-priority tasks preempt the owner. The high-priority task is indirectly delayed by medium-priority work.

High-priority Sensor -> waits for shared lock

Low-priority Storage -> owns shared lock

Medium-priority Comm -> repeatedly preempts Storage

Result: Sensor misses deadline

FreeRTOS mutexes provide priority inheritance, temporarily raising the resource owner's priority. V3's stronger strategy is single ownership: the storage task owns the file, and the sensor task owns acquisition, so the critical data path avoids broad shared locks.

# 7. Constructing app_core.c Step by Step

## 7.1 Create RTOS Object Handles

static EventGroupHandle_t s_events;

static QueueHandle_t s_sample_queue;

static QueueHandle_t s_command_queue;

static SemaphoreHandle_t s_status_lock;

static gptimer_handle_t s_collection_timer;

static TaskHandle_t s_sensor_task_handle;

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| s_events | Stores persistent recording/fault bits. | Several tasks must observe shared system state. |
| s_sample_queue | Transfers fixed-size batches from sensor to storage. | Separates timing and ownership. |
| s_command_queue | Transfers typed external commands to supervisor. | Serializes application state changes. |
| s_status_lock | Mutex protecting status snapshot. | Prevents partial concurrent reads/writes. |
| s_collection_timer | Handle for GPTimer lifecycle. | Application controls periodic release. |
| s_sensor_task_handle | Identifies sensor task for direct notifications. | Fast one-to-one ISR/task communication. |

## 7.2 Fault Latching

taskENTER_CRITICAL(&s_state_lock);

s_fault_mask |= (uint32_t)fault;

s_state = APP_STATE_FAULT;

taskEXIT_CRITICAL(&s_state_lock);

xEventGroupClearBits(s_events, APP_EVENT_RECORDING);

collection_timer_stop();

xEventGroupSetBits(s_events, APP_EVENT_FATAL_FAULT);

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| ENTER_CRITICAL | Acquires short cross-core spinlock. | State and fault mask must change atomically. |
| fault_mask ¦= | Preserves every observed fault category. | Postmortem diagnosis needs cumulative evidence. |
| state = FAULT | Moves explicit application state. | Behavior becomes understandable and testable. |
| EXIT_CRITICAL | Releases spinlock before RTOS calls. | Never block or call complex services inside critical section. |
| clear RECORDING | Tells tasks recording is no longer allowed. | Integrity fault invalidates continued acquisition. |
| stop timer | Prevents future collection releases. | Fault state should not restart acquisition. |
| set FATAL_FAULT | Broadcasts persistent fault condition. | Supervisor/commands can reject unsafe actions. |

## 7.3 ISR-to-Task Communication

BaseType_t higher_priority_task_woken = pdFALSE;

vTaskNotifyGiveFromISR(s_sensor_task_handle,

&higher_priority_task_woken);

return higher_priority_task_woken == pdTRUE;

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| higher_priority_task_woken | Receives whether notification made a higher-priority task Ready. | Kernel needs to know whether immediate rescheduling is useful. |
| FromISR API | Uses interrupt-safe notification operation. | Normal task APIs may block or use unsafe kernel paths. |
| Return bool | Requests yield after ISR when needed. | Minimizes response latency without always forcing a switch. |

## 7.4 Sensor Task Line-by-Line

for (;;) {

bool recording = event_recording_is_set();

if (!running) {

ulTaskNotifyTake(pdTRUE, portMAX_DELAY);

recording = event_recording_is_set();

}

if (recording && !running) {

sensor_service_start();

running = true;

window_deadline_us = now_us() + window_us;

}

if (running && (!recording || now_us() >= deadline)) {

sensor_service_stop();

running = false;

}

if (running) {

sensor_service_read_batch(&batch);

xQueueSend(sample_queue, &batch, timeout);

}

}

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| for (;;) | Creates permanent RTOS task loop. | Tasks normally live for firmware lifetime. |
| read recording event | Observes product state. | Event group is shared persistent condition. |
| if !running | Only wait when no window is active. | Prevents blocking while acquisition should continue. |
| ulTaskNotifyTake | Blocks indefinitely until START/timer/STOP notification. | Zero CPU use while idle. |
| re-read event | Validates reason after wakeup. | Notification is wakeup, event bit is authoritative state. |
| service_start | Starts sensors through service contract. | Task does not know registers. |
| deadline | Creates absolute monotonic stop time. | Avoids accumulated delay drift. |
| service_stop | Ends active window. | Controls power and clean sensor state. |
| read_batch | Obtains paired, timestamped samples. | Service owns acquisition policy. |
| queue send | Transfers batch to storage owner. | Sensor task never performs file I/O. |

## 7.5 Storage Task Line-by-Line

if (recording && !session_open) {

session_open = storage_service_begin_session(...) == ESP_OK;

}

if (xQueueReceive(s_sample_queue, &batch, timeout) == pdPASS &&

session_open) {

storage_service_write_batch(&batch);

}

if (!recording && session_open) {

storage_service_end_session();

session_open = false;

xQueueReset(s_sample_queue);

}

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| recording && !open | Opens exactly one file at session start. | Avoids repeated opens and clarifies lifecycle. |
| xQueueReceive | Blocks waiting for producer data. | Storage task consumes no CPU when no batch exists. |
| session_open | Writes only to valid owned session. | Prevents invalid file access. |
| write_batch | Delegates file formatting and integrity. | Task controls flow; service controls storage mechanism. |
| !recording && open | Detects session end. | Persistent event state drives lifecycle. |
| end_session | Writes footer, syncs, and closes. | Finalizes integrity metadata and durability. |
| xQueueReset | Removes stale queued batches. | Next session must not receive previous-session data. |

## 7.6 Supervisor Task

The supervisor is the policy owner. It receives typed commands, changes application state, refreshes health, publishes a protected snapshot, reports stack margins, and drives status LEDs. Keeping commands in one task prevents callbacks from racing to change state.

## 7.7 Startup and Task Creation

s_events = xEventGroupCreate();

s_sample_queue = xQueueCreate(...);

s_command_queue = xQueueCreate(...);

s_status_lock = xSemaphoreCreateMutex();

xTaskCreatePinnedToCore(sensor_task, "sensor", stack, NULL,

priority, &handle, core);

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| Create event group | Allocates persistent state bits. | Tasks need shared recording/fault conditions. |
| Create queues | Allocates bounded typed communication channels. | Communication contracts exist before tasks run. |
| Create mutex | Allocates status protection. | Status readers/writers require synchronization. |
| CreatePinnedToCore | Creates task with name, stack, priority, handle, and affinity. | Makes scheduling and diagnostics explicit. |
| Check return | Converts allocation failure into startup failure. | A partially created RTOS system is unsafe. |

# 8. Scheduling Scenarios: What Actually Runs?

## 8.1 Idle System

Sensor task is Blocked on task notification.

Storage task is Blocked on queue receive with timeout.

Supervisor task waits on command queue with timeout.

Communication task waits for UART input.

FreeRTOS idle tasks run, allowing housekeeping and watchdog behavior.

## 8.2 START Command

Communication task parses START and sends app_command_t to command queue.

Supervisor becomes Ready, receives command, sets recording event.

Supervisor notifies sensor task and starts GPTimer.

Sensor task becomes Ready at priority 8 and preempts lower-priority work.

Sensor task starts sensors and begins producing batches.

Storage task wakes when batches arrive and writes them whenever sensor task blocks/yields.

## 8.3 GPTimer Alarm

Hardware alarm interrupts current task.

ISR sends direct notification using vTaskNotifyGiveFromISR.

ISR reports whether the higher-priority sensor task was woken.

Scheduler may context-switch to sensor task immediately after ISR.

Sensor task validates recording state before starting a window.

## 8.4 Storage Stall

Storage task blocks inside a slow SD operation.

Sensor task continues producing because it has higher priority and a queue decouples it.

Queue depth increases.

If storage recovers, it drains queued batches.

If queue fills, sensor task records dropped batch and latches an integrity fault.

## 8.5 STOP Command

Supervisor clears recording event and stops timer.

Stopping timer notifies sensor task so it cannot remain blocked forever.

Sensor task observes recording false and stops sensors.

Storage task observes recording false, finalizes file, and clears stale queue entries.

System returns to IDLE if no fault is latched.

# 9. Alternatives, Trade-Offs, and What an Experienced Engineer Considers

| Decision | Chosen Strategy | Alternative | Why Chosen |
| --- | --- | --- | --- |
| Timer wakeup | Direct task notification | Binary semaphore/event bit | One known task; notification is lighter. |
| Storage access | Single-owner task | Mutex around file API | Avoids contention and priority inversion. |
| Sample transfer | Copy fixed batch through queue | Shared ring buffer/zero-copy | Simpler ownership; adequate baseline. |
| Sensor timing | Task reads batch with deadline | All work in ISR | I2C is not suitable for ISR context. |
| Fault behavior | Stop on integrity fault | Continue degraded | Research data must not silently appear complete. |
| CPU clock | Fixed 240 MHz | Dynamic power management | Predictable characterization baseline. |

## 9.1 When a Ring Buffer Would Be Better

A ring buffer or zero-copy pool can reduce copying at high sample rates. It introduces more ownership complexity, producer/consumer index synchronization, wrap-around handling, and recovery rules. Use it only when measurements show queue-copy overhead is a problem.

## 9.2 When More Tasks Are Not Better

Creating one task per small function increases stacks, context switches, synchronization, and debugging complexity. Create a task when work needs independent blocking, priority, deadline, or ownership. Otherwise use a function called by an owning task.

## 9.3 Why Event Bits Are Not Commands

A bit stores state, not history or payload. Two START events can collapse into one bit. Commands need a queue because every command should be consumed once and may contain data.

# 10. Add Store-and-Forward Google Drive Upload Without Delaying Collection

Cloud upload is intentionally added after reliable local collection and storage. WiFi latency is unbounded compared with sensor FIFO deadlines. The experienced-engineer strategy is to make upload best-effort and collection-critical.

## 10.1 Extend the Task Model

#define APP_SENSOR_TASK_PRIORITY       8

#define APP_STORAGE_TASK_PRIORITY      6

#define APP_SUPERVISOR_TASK_PRIORITY   4

#define APP_COMM_TASK_PRIORITY         3

#define APP_UPLOAD_TASK_PRIORITY       2

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| Sensor priority 8 | Protects FIFO-draining deadline. | Sample loss is permanent. |
| Storage priority 6 | Persists data before background work. | Local durability is required before upload. |
| Upload priority 2 | Runs only when higher-priority work is blocked. | Slow WiFi must not delay collection. |

## 10.2 File Lifecycle

session_123.part       -> active recording; never upload

session_123.ready      -> closed/synced; eligible for upload

session_123.uploading  -> resumable transfer in progress

session_123.uploaded   -> Drive success verified

| Code / Line | Objective and Mechanism | Why This Strategy |
| --- | --- | --- |
| .part | Marks an incomplete or active file. | Upload must never read a changing recording file. |
| .ready | Marks verified local durability. | Only finalized files enter upload backlog. |
| .uploading | Stores active transfer state. | Supports reboot/disconnect recovery. |
| .uploaded | Stores verified Drive completion. | Prevents accidental deletion before confirmation. |

## 10.3 Upload Task State Machine

WAIT_FOR_WIFI

-> FIND_OLDEST_READY_FILE

-> REQUEST_STORAGE_CHUNK

-> SEND_CHUNK

-> SAVE_CONFIRMED_OFFSET

-> VERIFY_REMOTE_FILE

-> MARK_UPLOADED

On failure, the task saves progress, returns the file to a recoverable state, waits using exponential backoff with jitter, and retries later. Recording state is never cleared by a network failure.

## 10.4 Keep Storage as the Only Filesystem Owner

Upload Task -> upload_read_request_queue -> Storage Task

Storage Task -> upload_chunk_queue       -> Upload Task

The upload task should not independently scan, rename, or read SD while storage writes recording data. Instead, it requests bounded chunks. The storage task always services sample writes first and upload reads only when the critical path has capacity.

if (xQueueReceive(sample_queue, &batch, 0) == pdPASS) {

storage_service_write_batch(&batch);       /* first priority */

} else if (xQueueReceive(upload_request_queue, &request, 0) == pdPASS) {

storage_service_read_upload_chunk(&request);

}

## 10.5 Persistent Upload Metadata

| Field | Purpose |
| --- | --- |
| session_id | Globally unique idempotency key; prevents duplicate Drive files. |
| file checksum / rows | Verifies local and remote object integrity. |
| upload state | Allows restart after reboot. |
| attempt count / next retry | Controls exponential backoff. |
| confirmed byte offset | Resumes a large partial upload. |
| Drive file ID | Records verified remote identity. |

## 10.6 Weak Connectivity and Mare Movement

Mare leaves WiFi coverage; upload pauses and completed files remain .ready on SD.

Sensor and storage tasks continue at full priority.

Connectivity task retries with increasing delay and jitter.

Mare returns; pending files upload oldest-first in the background.

If WiFi becomes weak during transfer, progress is persisted and resumed later.

## 10.7 Storage-Full Safety Policy

Size SD for worst-case time outside coverage plus safety margin.

Delete only confirmed uploaded files according to retention policy.

Never delete .part, .ready, .uploading, or unverified data automatically.

Publish backlog and low-space warnings.

Stop safely before overwriting unuploaded research data.

Core reasoning
Upload speed is recoverable because data remains local. Sensor-sample loss is permanent. Priorities, queues, and ownership must reflect this asymmetry.

# 11. Verification: Prove the Firmware Instead of Trusting It

## 10.1 Code-Level Checks

Compiler warnings and static analysis.

clang-format consistency.

Architecture contract checks: pins centralized, registers outside services, ISR remains lightweight.

Unit tests for checksum, parsing, state transitions, and driver logic.

## 10.2 Timing Measurements

| Measurement | How | Acceptance Question |
| --- | --- | --- |
| ISR-to-task latency | GPIO toggle/trace around ISR and task wake | Is worst-case latency below acquisition requirement? |
| Batch execution time | Timestamp before/after sensor_service_read_batch | Can FIFO be drained under worst load? |
| Queue high-water | Record maximum queue occupancy | Does chosen length absorb expected SD stalls? |
| Storage latency | Measure write/flush/fsync distribution | Can average consumer throughput exceed producer? |
| Stack margin | uxTaskGetStackHighWaterMark | Is approved safety margin preserved? |

## 10.3 Fault-Injection Tests

Disconnect PPG1/PPG2 during startup and recording.

Hold I2C bus or force transaction timeouts.

Remove/fill/corrupt SD card.

Delay storage until queue overflow.

Power-cycle during write, sync, and close.

Generate maximum communication traffic during acquisition.

Run long soak tests while checking memory, stacks, counters, and data continuity.

Disconnect/reconnect WiFi repeatedly while upload backlog grows and drains.

Reboot during upload and confirm resumable/idempotent recovery.

Fill SD with pending files and prove that unuploaded data is never deleted.

Interview-level conclusion
Architecture is a hypothesis. Professional firmware engineering closes the loop with instrumentation, worst-case measurements, fault injection, documented acceptance criteria, and release evidence.

# 12. Interview and Study Quick Reference

| Question | Concise Experienced Answer |
| --- | --- |
| Task vs thread? | FreeRTOS task is a thread-like schedulable unit with private stack and shared address space; it is not an isolated process. |
| Semaphore vs mutex? | Semaphore signals/counts; mutex protects an owned resource and supports priority inheritance. |
| Queue vs event group? | Queue transfers each payload once; event group broadcasts persistent state bits. |
| Why notification? | Fast one-to-one wakeup from ISR/task without a separate semaphore object. |
| Priority inversion? | High-priority task waits behind low owner while medium tasks delay that owner; avoid sharing or use mutex inheritance. |
| Context switch? | Kernel saves current task context and restores another when scheduling requires it. |
| Why RTOS? | Different deadlines and blocking behavior require independent tasks, priorities, and explicit communication. |
| Why minimal ISR? | Interrupts preempt tasks; long/blocking work increases latency and violates ISR safety. |
| How prove real time? | Measure worst-case execution, blocking, wake latency, queue depth, overflow, CPU, and stack margins. |
| Why low upload priority? | WiFi delay is recoverable because files remain local; sensor loss is permanent. |

# 13. Final Mental Model

Build from requirements downward and evidence upward. Requirements define deadlines and integrity. Deadlines define task priorities. Ownership defines task boundaries. Communication needs define queues, notifications, events, and mutexes. Drivers isolate mechanisms; services express capabilities; app_core expresses policy. Interrupts release urgent work but do not perform complex work. Faults stop unsafe behavior. Measurements and tests prove the design.

Five questions for every code decision
What requirement caused this? Who owns the resource? What can block? What happens on failure? What measurement proves it works?